# EnergyLens — Reality Gap & Data-Sourcing Plan

Carbon Accounting, EPC, CRREM, EU Regulation + the full data-honesty layer

Ali Mert Özdemir

2026-06-08

# Purpose / Amaç

Companion to EnergyLens_Study_Reference_EN.pdf — specifically Part II (Carbon Accounting, EPC, CRREM & EU Regulation) and Part V (the honesty layer).

For every number in EnergyLens that is currently an assumption / indicative / proxy value, this document gives three things:

- Now — what the platform does today (the honest current state).

- Should be — what the real standard or methodology requires, exactly.

- What to access — the real institution, dataset, API or agreement we must connect to make the number production-grade — with the access route, the licence, and the cost.

All regulatory dates, factors and data-source facts below were verified against primary sources in June 2026 (sources listed in §C). Where a value is a range or still a draft, it says so — nothing is invented.

tr — Bu doküman ne işe yarar: Worksheet’te “şu an indikatif / varsayım / proxy” diye işaretlediğim her sayı için → (1) şu an ne yapıyoruz, (2) gerçekte tam olarak nasıl olmalı, (3) bunu gerçeğe çevirmek için hangi kurumdan / API’den veri çekeriz veya hangi anlaşmayı yaparız (erişim yolu + lisans + maliyet). Hocaya da, freelance müşterisine de aynı tabloyu gösterebilirsin. Önce kısa tablo (§A), sonra her satırın detayı (§B), sonra kaynak rehberi (§C), en sonda “ne söylemeli” (§D).

# §A — At-a-glance comparison table

The short table. Read top-to-bottom: the first 6 rows are Part II (carbon / EPC / CRREM / regulation); rows 7–11 are the rest of the honesty layer (data realism across the platform). “Effort” = how hard it is for us to close the gap, not how important it is.

| # | Item | Now (what we do) | Should be (the real standard) | What we must access — who · route · cost | Effort |
| --- | --- | --- | --- | --- | --- |
| 1 | Scope 3 emissions | (Scope1+Scope2) × 8% screening, flagged scope3_disclosure_grade = False | GHG Protocol Scope 3 Standard — calculate the material categories from activity data × emission factors (buildings: Cat 13 downstream leased / tenant energy, Cat 1 embodied carbon, Cat 2 capital goods) | Tenant/sub-meter energy data (building owner, contractual) · emission-factor DB: DESNZ/DEFRA (free), ADEME Base Empreinte (free), ecoinvent (paid), EXIOBASE (free spend-based) · or Climatiq API (freemium) | High |
| 2 | Market-based Scope 2 | Supplier-factor column exists but no contract data → equals location-based | GHG Protocol Scope 2 Guidance dual reporting: market-based from supplier-specific factor / Guarantees of Origin; residual mix for untracked supply | Each building’s electricity contract + GoO certificates (utility/supplier) · AIB European Residual Mix for the residual factor (free, annual) | Medium |
| 3 | Scope 1 (gas + refrigerant) | Gas proxied from heating-share if no meter; refrigerant leakage excluded | Metered fuel × fuel factor; refrigerant fugitive = annual top-up (kg) × GWP (IPCC AR5/AR6). EU F-Gas Reg 2024/573 governs logs/leak checks | Gas meter / utility bills (owner) · refrigerant service logs / F-Gas logbook (HVAC contractor) · GWP values IPCC AR6 (free) · fuel factors DEFRA/DESNZ (free) | Medium |
| 4 | Grid emission factors (non-DE/TR) | DE (UBA 363 g/kWh, 2024) + TR (0.442) real & year-indexed; other countries IEA placeholder | Official annual location-based factor per country | EEA “GHG emission intensity of electricity” (free, per-country, EU) · IEA Emission Factors (paid, global) · ElectricityMaps (live/operational, freemium+commercial) | Low |
| 5 | EPC ratings | Synthetic energy_certificate for B001–B010 | Actual EPC from the building’s Energieausweis / national register | UK EPC Open Data (free API/bulk) · DE: no central register yet → due ~May 2026 (EPBD Art 22); until then per-building from owner/issuer · EU Building Stock Observatory (aggregate only) | Medium |
| 6 | CRREM pathways | Indicative 1.5 °C anchors in one swappable module | Official CRREM v2 pathways + emission factors, compared on Scope 1+2 basis | CRREM Library — open-access from ~1 Jul 2026 (free pathways + factors + Misalignment-Year guidance) · embedding into a commercial product sold to clients = CRREM License Partner agreement | Low (data) / Med (commercial embed) |
| 7 | Regulatory facts/dates | “Current as of mid-2026”; Battery Reg mis-numbered 2023/1670 in app | Verified, sourced, and the app number corrected to 2023/1542 | EUR-Lex (free) · EFRAG (ESRS) · national transposition trackers — see §B7 for the verified list | Low |
| 8 | Underlying building data | Consumption/solar/battery synthetic for 10 buildings; weather/price/grid-CO₂ real | Metered data from a real pilot building | A pilot (e.g. BSBI campus) · smart-meter feed (DE Smart Meter Gateway / utility portal) · BMS export (BACnet/Modbus) · utility-bill API | High |
| 9 | HVAC split | Heating/cooling/ventilation modeled (type coeff × HDD/CDD) | Sub-metered HVAC circuits | Building sub-meters / BMS points (facility) — a Tier-3 upsell | Medium |
| 10 | Forecast accuracy | MAPE is in-sample | Rolling-origin cross-validation (Prophet cross_validation) | Nothing external — code/method fix (more history helps) | Low |
| 11 | Battery ROI savings | Upper-bound (all discharge valued at peak tariff) | Price each discharged kWh by the hour it displaces (hourly dispatch vs hourly price) | ENTSO-E hourly day-ahead prices (free — we already pull this) — code/method fix | Low–Med |

tr — tabloyu nasıl kullan: Hocaya tek sayfa olarak göster. Sol taraf “dürüst şu an”, orta “gerçek standart”, sağ “neye erişmemiz lazım”. En kritik 3 satır: 1 (Scope 3), 2 (market-based S2), 6 (CRREM) — bunlar enerji/ESG akademisyeninin ilk soracağı yerler. Satır 7 (regülasyon) bir düzeltme içeriyor: app’te 2023/1670 yazan yerleri 2023/1542 yap.

# §B — Detailed reference (per item)

Each item: the gap in one line, what “real” means, the exact source + how to connect, and the honest sentence to say.

## B1 — Scope 3 emissions

The gap. We report Scope 3 as a flat 8 % of (Scope 1 + Scope 2) and explicitly flag it scope3_disclosure_grade = False. This is a screening placeholder, not a disclosure number.

What “real” means. The GHG Protocol Corporate Value Chain (Scope 3) Standard defines 15 categories. You do not report all 15 — you do a materiality screen, then calculate the material ones from activity data × emission factor. For a building portfolio the material categories are almost always:

- Category 13 — Downstream leased assets (the energy your tenants use in space you own and lease out). For most landlords this is the single biggest Scope 3 bucket.

- Category 1 — Purchased goods & services and Category 2 — Capital goods (the embodied carbon of construction materials and major refits).

- Plus Cat 5 (waste), Cat 6 (business travel), Cat 7 (commuting) where data exists.

Exact sources + how to connect:

- Activity data (the hard part): tenant electricity/heat consumption — obtained via green-lease clauses, tenant sub-metering, or the utility. This is a data-access agreement, not an API.

- Emission factors (the easy part):

  - UK DESNZ / DEFRA “Greenhouse gas reporting: conversion factors” — free, updated annually, the de-facto global default for activity-based factors.

  - ADEME Base Empreinte / Base Carbone (France) — free, official, large database.

  - ecoinvent — the gold-standard LCA database for embodied carbon (Cat 1/2) — paid licence (~€for a commercial seat).

  - EXIOBASE — free environmentally-extended input-output DB for spend-based estimates.

  - Climatiq — a commercial API that aggregates DEFRA/IEA/etc. — freemium, good for a productized integration.

Say this: “Scope 3 is currently an 8 % screening placeholder, clearly flagged as not disclosure-grade. To make it real I model the material categories — Category 13 tenant energy and Category 1 embodied carbon — from activity data times DEFRA/ADEME factors. The blocker is not the method, it’s getting tenant data via green-lease clauses.”

tr: Scope 3 = şu an %8 screening (etiketli, disclosure değil). Gerçeği: GHG Protocol Scope 3 Standardı, sadece material kategoriler (binalarda Kat 13 kiracı enerjisi + Kat 1 gömülü karbon). Faktör kolay (DEFRA/ADEME ücretsiz, ecoinvent paralı); zor olan kiracı verisi (yeşil kira sözleşmesi).

## B2 — Market-based Scope 2

The gap. The engine has a supplier_ef column, but with no real contract data it returns the location-based number twice.

What “real” means. The GHG Protocol Scope 2 Guidance requires dual reporting: location-based (grid average) and market-based (your actual procurement). Market-based uses, in order of preference: a supplier-specific emission factor, an Energy Attribute Certificate (in Europe a Guarantee of Origin, GoO), or — for the electricity you didn’t cover with a certificate — the residual mix.

Exact sources + how to connect:

- Supplier factor / GoOs — from each building’s electricity supply contract and the Guarantee-of-Origin certificates the utility issues. This is a per-building document, not an API.

- AIB European Residual Mix (Association of Issuing Bodies) — the authoritative residual-mix factor for European countries, published free every year at aib-net.org/facts/european-residual-mix, and explicitly aligned to the GHG Protocol market-based requirement.

Say this: “The market-based mechanism is built; it just needs each building’s supply contract and its Guarantees of Origin. For the uncovered portion I use the AIB residual mix, which is the European reference factor for exactly this purpose. Until I have the contracts, market-based equals location-based — and I say so.”

tr: Market-based S2 mekanizması hazır; gerçek tedarikçi sözleşmesi + GoO sertifikası lazım, kalanı için AIB Residual Mix (ücretsiz, yıllık, Avrupa referansı). Sözleşme gelene kadar location-based’e eşit — bunu dürüstçe söyle.

## B3 — Scope 1 (gas + refrigerant)

The gap. Where there’s no gas meter we proxy Scope 1 from a heating-share of electricity, and we exclude refrigerant leakage entirely. Refrigerants (R-410A, R-32) have very high global-warming potential and are the classic forgotten Scope 1 source.

What “real” means.

- Stationary combustion (gas/oil boilers): metered fuel use × a fuel emission factor.

- Fugitive / refrigerant emissions: annual_refrigerant_topup_kg × GWP. The top-up mass is the refrigerant a contractor adds to recharge a system — i.e. what leaked. GWP comes from IPCC (AR5 or AR6 100-year values; e.g. R-410A ≈ 2088). The EU F-Gas Regulation (EU) 2024/573 (in force 11 Mar 2024) mandates leak checks and logbooks — so the data exists on site.

Exact sources + how to connect:

- Gas/fuel data: the building’s gas meter / utility bills (owner).

- Refrigerant data: the F-Gas logbook / maintenance records held by the HVAC service contractor (mandatory under 2024/573).

- GWP values: IPCC AR6 (free, authoritative).

- Fuel factors: DEFRA/DESNZ (free).

Say this: “Scope 1 is solid where there’s a gas meter. The honest gap is refrigerant fugitive emissions — I exclude them today rather than fake them. The data exists: the F-Gas logbook the contractor must keep under EU 2024/573, multiplied by the IPCC GWP. That’s a clean next step.”

tr: Scope 1 gaz sayacı varsa sağlam. Açık: soğutucu gaz kaçağı (şu an hariç — uydurmuyorum). Gerçeği: yıllık dolum (kg) × IPCC GWP; veri zaten F-Gas logbook’ta (EU 2024/573 zorunlu kılıyor).

## B4 — Grid emission factors (countries beyond DE/TR)

The gap. Germany (UBA, year-indexed: 2022 = 433, 2023 = 386, 2024 = 363 g CO₂/kWh) and Turkey (0.442) are real and sourced. Other countries are IEA placeholders.

What “real” means. A per-country, annual, location-based factor, ideally consumption-based (includes imports/T&D losses) for disclosure.

Exact sources + how to connect (pick by need):

- EEA — “Greenhouse gas emission intensity of electricity generation” — free, official, per-country for Europe (the EU average fell ~9 % in 2024 vs 2023). Best free option to replace placeholders across the EU.

- IEA Emission Factors — global, monthly/quarterly for 50+ countries — paid subscription (single/multi-user; the licence forbids third-party work — relevant if reselling).

- ElectricityMaps — live / operational grid intensity via API (freemium for non-commercial, commercial licence for production). Use for operational insight, not for the annual disclosure factor.

Keep the two apart: the disclosure factor stays annual-official (UBA/EEA/IEA). Live ElectricityMaps is for operational dashboards only — auditors want the fixed annual number.

tr: DE (UBA 363, 2024) + TR (0.442) gerçek; diğerleri placeholder. Ücretsiz çözüm = EEA (Avrupa, ülke-bazlı). Global için IEA (paralı). Canlı için ElectricityMaps — ama disclosure faktörü yıllık-resmî kalmalı.

## B5 — EPC ratings

The gap. The energy_certificate (A+→G) for the 10 demo buildings is synthetic.

What “real” means. The building’s actual EPC (in Germany the Energieausweis, issued under GEG by a qualified issuer), pulled from a national register where one exists.

Exact sources + how to connect:

- UK — EPC Open Data (epc.opendatacommunities.org) — free, official, bulk download + API for England & Wales (Scotland separate). The cleanest open EPC dataset in Europe — good for a UK demo.

- Germany — no central register yet. Germany is the only large EU state without an EPC database; one is due ~May 2026 under EPBD Article 22, which also requires free access to the full EPC for owners/tenants/managers. Until it’s live, EPC is a per-building document from the owner or issuer.

- EU Building Stock Observatory — free but aggregate (national stock statistics), not per-building — useful for benchmarks, not for a specific building’s rating.

- Going forward, EPBD Art 22 forces every member state’s EPC database to be accessible and to feed the BSO annually — so per-country EPC access will improve through 2026–2027.

tr: EPC şu an sentetik. Gerçeği: binanın Energieausweis’i. UK ücretsiz açık veri (demo için ideal). Almanya’da merkezi kayıt yok → ~Mayıs 2026 (EPBD Md. 22) geliyor; o zamana kadar bina-bazlı sahibinden. EU BSO sadece toplu istatistik.

## B6 — CRREM pathways

The gap. Our decarbonization pathways are indicative 1.5 °C anchors, isolated in one module (PATHWAY_ANCHORS) so they can be swapped for official values.

What “real” means. The official CRREM v2 pathways + emission factors, with the comparison done on the correct scope (CRREM uses Scope 1+2, excludes Scope 3 — EnergyLens was already corrected to compare on this basis).

Exact sources + how to connect — note the 2026 change:

- The free CRREM Excel tool retires 1 July 2026. It is being replaced by an open-access “CRREM Library”: the pathways, emission factors and Misalignment-Year computation guidance are published free for open use. So the raw data to compute a correct stranding year becomes freely available.

- But embedding CRREM pathways/methodology into a commercial platform sold to clients still requires a CRREM License Partner agreement (crrem.org/license-partners). Nearly every major real-estate ESG platform is a License Partner.

- Practical plan: use the open CRREM Library values to replace our indicative anchors now (correct data, isolated module); take the License Partner step when EnergyLens is sold commercially.

Say this: “My pathways are indicative today. CRREM is opening its dataset as a free library mid-2026, so I’ll swap in the official curves — they drop into one isolated module. For commercial resale I’d take the CRREM License Partner agreement, which is the standard route every ESG platform uses.”

tr: Pathway’ler indikatif. CRREM Library (1 Tem 2026, ücretsiz) ile resmî eğrileri tek modüle takarım. Ticari satışta License Partner anlaşması (sektör standardı). Karşılaştırma Scope 1+2.

## B7 — Regulatory facts (verified June 2026) + the one correction

This is what an academic will check. All verified against primary sources this month:

- CSRD / Omnibus I — the simplification directive was published in the EU Official Journal on 26 February 2026 (Council adoption 24 Feb 2026; in force ~18 Mar 2026). It narrows mandatory scope to companies with > 1,000 employees AND > €450 m net turnover; the listed-SME wave is removed; the new scope applies for financial years from 1 January 2027. Consequence for positioning: many mid-caps the old material called “Wave 2, filing FY2025 in 2026” are now outside mandatory scope — so position EnergyLens as “ESRS-E1-aligned report-ready support” and point smaller firms to the voluntary VSME standard. Do not say “everyone must comply.”

- EPBD recast — Directive (EU) 2024/1275 — in force 28 May 2024; national transposition due 29 May 2026. Non-residential MEPS: member states set national standards by 1 January 2027; worst-performing 16 % renovated by 2030, 26 % by 2033; new buildings zero-emission from 2030.

- EU Battery Regulation — Regulation (EU) 2023/1542 ⚠️ the app says “2023/1670” in places — fix it. Industrial carbon-footprint declaration applies from 18 Feb 2026; the digital Battery Passport via QR from 18 Feb 2027 (industrial/EV/LMT > 2 kWh).

- EU Taxonomy — Activity 7.7 “Acquisition and ownership of buildings.” Original Climate Delegated Act: pre-2021 buildings qualify (climate-mitigation substantial contribution) with EPC A or top-15 % of national stock by operational primary energy demand; full alignment also needs DNSH + minimum safeguards. 2026 draft revision loosens this to EPC D or top-50 % — draft, verify on adoption. Never claim “Taxonomy-aligned” — only an indicative alignment signal for the SC criterion.

- EU F-Gas — Regulation (EU) 2024/573 (in force 11 Mar 2024): leak checks, recovery, logbooks — the basis for refrigerant Scope 1 (B3).

- EnEfG (Germany): ISO 50001 / audit for ≥ 250 employees; energy-management system if annual final energy > 7.5 GWh. GEG §71: heating installed from 2024 must be ≥ 65 % renewable (a heat pump satisfies it); min U-values (wall ≤ 0.24, roof ≤ 0.20, window ≤ 1.30 W/m²K).

Access: all of the above are on EUR-Lex (free full text) and EFRAG (ESRS standards, free).

tr: En kritik: CSRD kapsamı daraldı (>1000 çalışan & >€450M, FY2027) → “herkese zorunlu” DEME, “report-ready / VSME” de. Battery Reg numarasını app’te 2023/1542 yap. Taxonomy 7.7 2026 taslağı gevşetiyor (EPC D / top-50%, taslak). Hepsi EUR-Lex’te ücretsiz.

## B8 — Underlying building data (synthetic → real)

The gap. Building-level consumption / solar / battery data is synthetic for B001–B010. The external signals are already real: Open-Meteo/DWD weather (211k real hourly rows feeding real HDD/CDD), ENTSO-E prices, ElectricityMaps grid CO₂.

What “real” means. Metered time-series from at least one real building.

Exact sources + how to connect:

- A pilot building — the cleanest path (e.g. the BSBI campus, which doubles as the EXIST/academic story).

- Smart-meter data: in Germany via the Smart Meter Gateway rollout / the utility’s customer portal; elsewhere via the DSO/utility API.

- BMS export: BACnet/Modbus points from the building automation system (the IoT adapters already exist).

- Utility-bill ingestion: CSV/API from the energy supplier.

Honest framing: synthetic operational data is normal and legitimate for a pre-customer demo — the methods, factors and external signals are real. State it plainly; an academic respects “synthetic operational layer, real methodology and real external data.”

tr: Bina tüketim/solar/batarya sentetik (henüz müşteri yok); hava/fiyat/grid-CO₂ gerçek. Gerçeğe geçiş = pilot bina (BSBI kampüsü), akıllı sayaç / BMS export / fatura API’si.

## B9 — HVAC heating/cooling/ventilation split

The gap. The split is modeled from a building-type coefficient × the weather shape (HDD/CDD), so an office’s HVAC share is essentially pinned by its type. A facilities engineer spots this instantly.

Should be: sub-metered HVAC circuits (or dedicated BMS energy points).

Access: building sub-meters / BMS metering points (facility hardware) — frame as a Tier-3 upsell: “modeled split using the HDD/CDD method — sub-metering refines this.”

tr: HVAC split modellenmiş (sub-metered değil). Gerçeği: alt-sayaç / BMS noktası (Tier-3 upsell).

## B10 — Forecast accuracy metric

The gap. Forecast error (MAPE) is computed in-sample — testing the model on its own training data, which flatters accuracy.

Should be: rolling-origin cross-validation (Prophet’s cross_validation) so the reported MAPE is out-of-sample and honest.

Access: none external — a code/method fix; more history per building improves it.

tr: MAPE in-sample → rolling-origin cross-validation. Dış veri gerekmez, kod düzeltmesi.

## B11 — Battery ROI savings

The gap. Savings are an upper bound — all discharged energy is valued at the peak tariff and the demand saving is proportional to energy, not to peak-kW. So payback looks optimistic.

Should be: price each discharged kWh by the hour it actually displaces (hourly dispatch against the hourly price curve), and value demand-charge savings on the actual peak-kW reduction.

Access: ENTSO-E hourly day-ahead prices (free — already pulled) — a method/code fix, not new data.

tr: Batarya ROI upper-bound (hepsi peak tarifeden). Gerçeği: her deşarj kWh’ını yerine geçtiği saatin fiyatıyla değerle (ENTSO-E saatlik — zaten var). Kod düzeltmesi.

# §C — Data-source directory (institution · what · access · cost)

Quick reference — the real institutions, grouped. Use this when the professor asks “where would the data come from?” or when scoping a freelance build.

### Emission factors & carbon accounting

- UK DESNZ / DEFRA — GHG conversion factors — activity-based factors (fuel, refrigerant, travel). Free, annual. (global default)

- ADEME — Base Empreinte / Base Carbone (FR) — large official factor DB. Free.

- ecoinvent — LCA database for embodied carbon (Scope 3 Cat 1/2). Paid licence.

- EXIOBASE — environmentally-extended input-output (spend-based Scope 3). Free.

- Climatiq — API aggregating DEFRA/IEA/etc. for productized integration. Freemium.

- IPCC (AR5/AR6) — refrigerant & gas GWP values. Free.

- GHG Protocol — the Corporate, Scope 2 and Scope 3 standards + calculation guidance. Free (ghgprotocol.org).

- AIB — European Residual Mix — market-based Scope 2 residual factor. Free, annual (aib-net.org).

### Grid emission factors (electricity)

- UBA (Umweltbundesamt) — Germany annual grid factor (363 g/kWh, 2024). Free.

- EEA — per-country EU electricity carbon intensity. Free.

- IEA — Emission Factors — global, 50+ countries, monthly/quarterly. Paid subscription (resale-restricted).

- ElectricityMaps — live grid intensity API. Freemium (non-commercial) / commercial licence for production.

- TEİAŞ (TR) — Turkish grid operator; single canonical factor (~0.442). (less granular than UBA)

### Prices & weather

- ENTSO-E Transparency Platform — EU day-ahead/hourly electricity prices + generation. Free API (email transparency@entsoe.eu, subject “Restful API access”).

- DWD Open Data — official German weather, free incl. commercial (legal mandate). Use this for the commercial build.

- Open-Meteo — free non-commercial only (CC BY 4.0, < 10k calls/day); commercial = paid subscription or self-host the open-source. (we use it now; switch to DWD/self-host for sale)

- EPİAŞ / EXIST — Şeffaflık Platformu (TR) — Turkish day-ahead market prices. Free.

- MGM (TR) — Turkish state meteorology. Weather for Turkish buildings.

### EPC, buildings & decarbonization

- UK EPC Open Data (epc.opendatacommunities.org) — free EPC dataset (England & Wales) + API.

- Germany central EPC register — due ~May 2026 (EPBD Art 22); until then per-building Energieausweis.

- EU Building Stock Observatory — free aggregate building-stock + EPC statistics.

- CRREM Library — open-access pathways + emission factors from ~1 Jul 2026 (free); License Partner for commercial embedding (crrem.org).

### Regulation (primary text)

- EUR-Lex — full text of every directive/regulation (CSRD/Omnibus, EPBD 2024/1275, Battery 2023/1542, F-Gas 2024/573, Taxonomy). Free.

- EFRAG — the ESRS standards (incl. ESRS E1) + the voluntary VSME. Free.

### Real building data (to replace synthetic)

- Pilot building / BSBI campus — metered consumption.

- Smart-meter feeds — DE Smart Meter Gateway / utility portals; DSO APIs elsewhere.

- BMS export — BACnet/Modbus (adapters already built).

# §D — What to say (professor + freelance clients)

The honesty framing (memorize the shape, not the words):

- “The methodology and the external data are real; the operational layer is synthetic until a pilot. Here’s exactly what I’d connect to make each number production-grade.” — then point at §A.

- “Scope 1/2 are auditable today with row-level lineage from a sourced factor to the disclosed kg. Scope 3 is a flagged screening estimate, and market-based Scope 2 is built but waiting on supplier data — I label both honestly rather than overstate.”

- “CRREM is indicative now; the official curves become a free open library in mid-2026 and drop into one module, with a License Partner agreement for commercial resale.”

- “I keep the disclosure factor annual-official and use live grid data only for operational insight — because that’s what an auditor expects.”

Why this wins with an academic: naming the exact gap and the exact source to close it is the strongest possible credibility signal. An academic rewards “I know precisely what isn’t real yet and precisely how to fix it” far more than a polished claim that everything is finished.

Freelance angle (same content, commercial spin): this table is a productized scope-of-work. Each “What to access” cell is a paid integration you can quote — “connect your AIB residual mix + GoOs for audit-ready market-based Scope 2,” “wire your DEFRA factors for Scope 3 Cat 13,” “swap in licensed CRREM curves.” You sell closing the gap, building by building.

tr — özet: Hocaya güç veren şey “her şey bitti” demek değil, “neyin gerçek olmadığını ve onu kapatmak için tam olarak hangi kaynağa bağlanacağımı biliyorum” demek. Aynı tablo freelance’te satış kapsamı: her “neye erişmemiz lazım” hücresi ücretlendirebileceğin bir entegrasyon.

Verified June 2026 against EUR-Lex, EFRAG, UBA, EEA, IEA, AIB, CRREM Foundation, DWD, Open-Meteo, DESNZ/DEFRA, ADEME, ENTSO-E and the UK EPC register. Regulatory dates — re-check on EUR-Lex right before the meeting; the EU Taxonomy 7.7 loosening (EPC D / top-50 %) is a 2026 draft, not yet adopted.